SIMULATION EXPERIMENT – 4

Design and Comparative Analysis of Butterworth, Chebyshev and Elliptic IIR Filters

Using MATLAB

Program:

clc;

clear;

close all;

%% =========================================================

% DESIGN AND COMPARATIVE ANALYSIS OF

% BUTTERWORTH, CHEBYSHEV-I AND ELLIPTIC IIR FILTERS

% =========================================================

%% 1. INPUT PARAMETERS

Fs = 1000; % Sampling frequency (Hz)

Fp = 100; % Passband edge (Hz)

Fst = 150; % Stopband edge (Hz)

Rp = 1; % Passband ripple (dB)

Rs = 40; % Stopband attenuation (dB)

% Normalized frequencies

Wp = Fp/(Fs/2);

Wst = Fst/(Fs/2);

%% =========================================================

% 2. CHEBYSHEV TYPE-I FORMULA

% =========================================================

epsilon = sqrt(10^(Rp/10)-1);

N_formula = acosh(sqrt((10^(Rs/10)-1)/(10^(Rp/10)-1))) ...

/ acosh(Fst/Fp);

N_theory = ceil(N_formula);

%% =========================================================

% 3. BUTTERWORTH FILTER

% =========================================================

[Nb,Wb] = buttord(Wp,Wst,Rp,Rs);

[bB,aB] = butter(Nb,Wb,'low');

%% =========================================================

% 4. CHEBYSHEV TYPE-I FILTER

% =========================================================

[Nc,Wc] = cheb1ord(Wp,Wst,Rp,Rs);

[bC,aC] = cheby1(Nc,Rp,Wc,'low');

%% =========================================================

% 5. ELLIPTIC FILTER

% =========================================================

[Ne,We] = ellipord(Wp,Wst,Rp,Rs);

[bE,aE] = ellip(Ne,Rp,Rs,We,'low');

%% =========================================================

% 6. DISPLAY INPUT PARAMETERS

% =========================================================

fprintf('\n====================================================\n');

fprintf(' IIR FILTER COMPARATIVE ANALYSIS\n');

fprintf('====================================================\n');

fprintf('\nINPUT PARAMETERS\n');

fprintf('Sampling Frequency = %.2f Hz\n',Fs);

fprintf('Passband Edge Frequency = %.2f Hz\n',Fp);

fprintf('Stopband Edge Frequency = %.2f Hz\n',Fst);

fprintf('Passband Ripple = %.2f dB\n',Rp);

fprintf('Stopband Attenuation = %.2f dB\n',Rs);

%% =========================================================

% 7. CHEBYSHEV FORMULA RESULTS

% =========================================================

fprintf('\nCHEBYSHEV TYPE-I FORMULA\n');

fprintf('Ripple factor epsilon = %.4f\n',epsilon);

fprintf('Calculated order = %.4f\n',N_formula);

fprintf('Theoretical minimum N = %d\n',N_theory);

%% =========================================================

% 8. FILTER ORDER COMPARISON

% =========================================================

fprintf('\n====================================================\n');

fprintf(' FILTER ORDER COMPARISON\n');

fprintf('====================================================\n');

fprintf('Butterworth Order = %d\n',Nb);

fprintf('Chebyshev Type-I Order = %d\n',Nc);

fprintf('Elliptic Order = %d\n',Ne);

%% =========================================================

% 9. COEFFICIENTS

% ==========================================================

fprintf('\n====================================================\n');

fprintf(' BUTTERWORTH COEFFICIENTS\n');

fprintf('====================================================\n');

fprintf('Numerator b =\n');

disp(bB);

fprintf('Denominator a =\n');

disp(aB);

fprintf('\n====================================================\n');

fprintf(' CHEBYSHEV TYPE-I COEFFICIENTS\n');

fprintf('====================================================\n');

fprintf('Numerator b =\n');

disp(bC);

fprintf('Denominator a =\n');

disp(aC);

fprintf('\n====================================================\n');

fprintf(' ELLIPTIC COEFFICIENTS\n');

fprintf('====================================================\n');

fprintf('Numerator b =\n');

disp(bE);

fprintf('Denominator a =\n');

disp(aE);

%% =========================================================

% 10. FREQUENCY RESPONSES

% ==========================================================

N = 4096;

[HB,f] = freqz(bB,aB,N,Fs);

[HC,~] = freqz(bC,aC,N,Fs);

[HE,~] = freqz(bE,aE,N,Fs);

MB = 20*log10(abs(HB)+eps);

MC = 20*log10(abs(HC)+eps);

ME = 20*log10(abs(HE)+eps);

%% =========================================================

% 11. PASSBAND RIPPLE

% ==========================================================

P = f <= Fp;

Ripple_B = max(MB(P))-min(MB(P));

Ripple_C = max(MC(P))-min(MC(P));

Ripple_E = max(ME(P))-min(ME(P));

%% =========================================================

% 12. STOPBAND ATTENUATION

% ==========================================================

S = f >= Fst;

Stop_B = -max(MB(S));

Stop_C = -max(MC(S));

Stop_E = -max(ME(S));

%% =========================================================

% 13. PERCENTAGE DEVIATION

% ==========================================================

Dev_C_Ripple = abs(Ripple_C-Rp)/Rp*100;

Dev_B_Stop = abs(Stop_B-Rs)/Rs*100;

Dev_C_Stop = abs(Stop_C-Rs)/Rs*100;

Dev_E_Stop = abs(Stop_E-Rs)/Rs*100;

%% =========================================================

% 14. TRANSITION CHARACTERISTICS

% ==========================================================

TransitionBW = Fst-Fp;

%% =========================================================

% 15. DISPLAY PERFORMANCE

% ==========================================================

fprintf('\n====================================================\n');

fprintf(' FILTER PERFORMANCE\n');

fprintf('====================================================\n');

fprintf('\nPASSBAND RIPPLE\n');

fprintf('Butterworth = %.4f dB\n',Ripple_B);

fprintf('Chebyshev Type-I = %.4f dB\n',Ripple_C);

fprintf('Elliptic = %.4f dB\n',Ripple_E);

fprintf('\nSTOPBAND ATTENUATION\n');

fprintf('Butterworth = %.4f dB\n',Stop_B);

fprintf('Chebyshev Type-I = %.4f dB\n',Stop_C);

fprintf('Elliptic = %.4f dB\n',Stop_E);

fprintf('\nCHEBYSHEV RIPPLE DEVIATION\n');

fprintf('Specified Ripple = %.2f dB\n',Rp);

fprintf('Measured Ripple = %.4f dB\n',Ripple_C);

fprintf('Percentage Deviation = %.4f %%\n',Dev_C_Ripple);

fprintf('\nTRANSITION CHARACTERISTICS\n');

fprintf('Passband Edge = %.2f Hz\n',Fp);

fprintf('Stopband Edge = %.2f Hz\n',Fst);

fprintf('Transition Band = %.2f Hz\n',TransitionBW);

%% =========================================================

% 16. MAGNITUDE RESPONSE

% ==========================================================

figure;

plot(f,MB,'LineWidth',1.5);

hold on;

plot(f,MC,'LineWidth',1.5);

plot(f,ME,'LineWidth',1.5);

grid on;

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

title('Comparison of IIR Filter Magnitude Responses');

legend('Butterworth','Chebyshev Type-I','Elliptic');

xlim([0 250]);

ylim([-80 5]);

xline(Fp,'--','Fp = 100 Hz');

xline(Fst,'--','Fst = 150 Hz');

yline(-Rp,'--','Rp = -1 dB');

yline(-Rs,'--','Rs = -40 dB');

hold off;

%% =========================================================

% 17. PHASE RESPONSE

% ==========================================================

Phase_B = unwrap(angle(HB))*180/pi;

Phase_C = unwrap(angle(HC))*180/pi;

Phase_E = unwrap(angle(HE))*180/pi;

figure;

plot(f,Phase_B,'LineWidth',1.2);

hold on;

plot(f,Phase_C,'LineWidth',1.2);

plot(f,Phase_E,'LineWidth',1.2);

grid on;

xlabel('Frequency (Hz)');

ylabel('Phase (degrees)');

title('Comparison of IIR Filter Phase Responses');

legend('Butterworth','Chebyshev Type-I','Elliptic');

xlim([0 250]);

hold off;

%% =========================================================

% 18. FILTER ORDER GRAPH

% ==========================================================

figure;

bar([Nb Nc Ne]);

set(gca,'XTick',1:3);

set(gca,'XTickLabel',...

{'Butterworth','Chebyshev-I','Elliptic'});

ylabel('Filter Order');

xlabel('Filter Type');

title('Comparison of IIR Filter Orders');

grid on;

Output

INPUT PARAMETERS

Sampling Frequency = 1000.00 Hz

Passband Edge Frequency = 100.00 Hz

Stopband Edge Frequency = 150.00 Hz

Passband Ripple = 1.00 dB

Stopband Attenuation = 40.00 dB

CHEBYSHEV TYPE-I FORMULA

Ripple factor epsilon = 0.5088

Calculated order = 6.2071

Theoretical minimum N = 7

====================================================

FILTER ORDER COMPARISON

====================================================

Butterworth Order = 12

Chebyshev Type-I Order = 6

Elliptic Order = 4

====================================================

BUTTERWORTH COEFFICIENTS

====================================================

Numerator b =

1.0e-03 *

Columns 1 through 4

0.0002 0.0027 0.0146 0.0488

Columns 5 through 8

0.1098 0.1757 0.2050 0.1757

Columns 9 through 12

0.1098 0.0488 0.0146 0.0027

Column 13

0.0002

Denominator a =

Columns 1 through 4

1.0000 -6.8844 22.4341 -45.5100

Columns 5 through 8

63.7725 -64.8501 48.9643 -27.6092

Columns 9 through 12

11.5218 -3.4662 0.7128 -0.0899

Column 13

0.0053

====================================================

CHEBYSHEV TYPE-I COEFFICIENTS

====================================================

Numerator b =

1.0e-03 *

Columns 1 through 4

0.0464 0.2782 0.6956 0.9274

Columns 5 through 7

0.6956 0.2782 0.0464

Denominator a =

Columns 1 through 4

1.0000 -4.8694 10.3807 -12.3367

Columns 5 through 7

8.5969 -3.3268 0.5586

====================================================

ELLIPTIC COEFFICIENTS

====================================================

Numerator b =

Columns 1 through 4

0.0197 -0.0171 0.0333 -0.0171

Column 5

0.0197

Denominator a =

Columns 1 through 4

1.0000 -3.0330 3.8118 -2.2911

Column 5

0.5554

====================================================

FILTER PERFORMANCE

====================================================

PASSBAND RIPPLE

Butterworth = 0.8033 dB

Chebyshev Type-I = 1.0000 dB

Elliptic = 1.0000 dB

STOPBAND ATTENUATION

Butterworth = 40.0198 dB

Chebyshev Type-I = 41.3365 dB

Elliptic = 40.0000 dB

CHEBYSHEV RIPPLE DEVIATION

Specified Ripple = 1.00 dB

Measured Ripple = 1.0000 dB

Percentage Deviation = 0.0000 %

TRANSITION CHARACTERISTICS

Passband Edge = 100.00 Hz

Stopband Edge = 150.00 Hz

Transition Band = 50.00 Hz

OUTPUT:

RESULT:

============================================
IIR FILTER COMPARATIVE ANALYSIS
============================================

INPUT PARAMETERS
Sampling Frequency = 1000.00 Hz
Passband Edge Frequency = 100.00 Hz
Stopband Edge Frequency = 150.00 Hz
Passband Ripple = 1.00 dB

Stopband Attenuation = 40.00 dB

CHEBYSHEV TYPE-I THEORETICAL ORDER
Ripple factor epsilon = 0.5088
Calculated order = 5.8507
Theoretical minimum order = 6

============================================
FILTER ORDER COMPARISON
============================================
Butterworth Order = 12
Chebyshev Type-I Order = 6
Elliptic Order = 4

============================================
BUTTERWORTH COEFFICIENTS
============================================
Numerator bB =
1.0e-03 *

0.0002 0.0027 0.0146 0.0488 0.1098 0.1757 0.2050 0.1757 0.1098 0.0488 0.0146 0.0027 0.0002

Denominator aB =
1.0000 -6.8844 22.4341 -45.5100 63.7725 -64.8501 48.9643 -27.6092 11.5218 -3.4662 0.7128 -0.0899 0.0053


============================================
CHEBYSHEV TYPE-I COEFFICIENTS
============================================
Numerator bC =
1.0e-03 *

0.0464 0.2782 0.6956 0.9274 0.6956 0.2782 0.0464

Denominator aC =
1.0000 -4.8694 10.3807 -12.3367 8.5969 -3.3268 0.5586


============================================
ELLIPTIC COEFFICIENTS
============================================
Numerator bE =
0.0197 -0.0171 0.0333 -0.0171 0.0197

Denominator aE =
1.0000 -3.0330 3.8118 -2.2911 0.5554

============================================
 FILTER PERFORMANCE
============================================

PASSBAND RIPPLE
Butterworth = 0.8033 dB
Chebyshev Type-I = 1.0000 dB
Elliptic = 1.0000 dB

STOPBAND ATTENUATION
Butterworth = 40.0198 dB
Chebyshev Type-I = 41.3365 dB
Elliptic = 40.0000 dB

CHEBYSHEV TYPE-I RIPPLE DEVIATION
Specified Ripple = 1.00 dB
Measured Ripple = 1.0000 dB
Percentage Deviation = 0.0000 %

STOPBAND ATTENUATION DEVIATION
Butterworth = 0.0494 %
Chebyshev Type-I = 3.3413 %
Elliptic = 0.0000 %

TRANSITION CHARACTERISTICS
Passband Edge = 100.00 Hz
Stopband Edge = 150.00 Hz
Transition Bandwidth = 50.00 Hz

============================================
 EXPERIMENT COMPLETED SUCCESSFULLY
============================================
Butterworth order = 12
Chebyshev Type-I order = 6
Elliptic order = 4

All three filter responses have been plotted.