SIMULATION EXPERIMENT – 1

1.Design, Realization and Performance Analysis of Butterworth IIR Low-Pass and

High-Pass Digital Filters Using MATLAB

Program Code:

clear;

clc;

close all;

% Design Parameters

Fs = 8000; % Sampling frequency in Hz

Ap = 1; % Passband ripple in dB

As = 40; % Stopband attenuation in dB

% 1. Low-Pass Filter Design

fp_lp = 1000; % LPF Passband frequency (Hz)

fs_lp = 1500; % LPF Stopband frequency (Hz)

wp_lp = fp_lp / (Fs/2);

ws_lp = fs_lp / (Fs/2);

[N_lp, Wn_lp] = buttord(wp_lp, ws_lp, Ap, As);

[b_lp, a_lp] = butter(N_lp, Wn_lp, 'low');

% 2. High-Pass Filter Design

fp_hp = 2500; % HPF Passband frequency (Hz)

fs_hp = 1800; % HPF Stopband frequency (Hz)

wp_hp = fp_hp / (Fs/2);

ws_hp = fs_hp / (Fs/2);

[N_hp, Wn_hp] = buttord(wp_hp, ws_hp, Ap, As);

[b_hp, a_hp] = butter(N_hp, Wn_hp, 'high');

% ==========================================

% DISPLAY NUMERICAL OUTPUTS IN COMMAND WINDOW

% ==========================================

fprintf('=== EXPERIMENT 1: BUTTERWORTH IIR FILTERS ===\n\n');

fprintf('--- LOW-PASS FILTER ---\n');

fprintf('Calculated Filter Order (N_lp): %d\n', N_lp);

fprintf('3-dB Cutoff Frequency (Wn_lp): %.4f Hz\n', Wn_lp * (Fs/2));

fprintf('Numerator Coefficients (b_lp):\n');

disp(b_lp);

fprintf('Denominator Coefficients (a_lp):\n');

disp(a_lp);

fprintf('--- HIGH-PASS FILTER ---\n');

fprintf('Calculated Filter Order (N_hp): %d\n', N_hp);

fprintf('3-dB Cutoff Frequency (Wn_hp): %.4f Hz\n', Wn_hp * (Fs/2));

fprintf('Numerator Coefficients (b_hp):\n');

disp(b_hp);

fprintf('Denominator Coefficients (a_hp):\n');

disp(a_hp);

% Frequency Response Calculation

[H_lp, f] = freqz(b_lp, a_lp, 1024, Fs);

[H_hp, ~] = freqz(b_hp, a_hp, 1024, Fs);

% Plotting

figure('Name', 'Butterworth IIR LPF and HPF');

subplot(2,2,1);

plot(f, 20*log10(abs(H_lp)), 'b', 'LineWidth', 1.5); grid on;

title(['Butterworth LPF Magnitude (Order N = ', num2str(N_lp), ')']);

xlabel('Frequency (Hz)'); ylabel('Magnitude (dB)'); ylim([-80 5]);

subplot(2,2,2);

plot(f, angle(H_lp)*180/pi, 'r', 'LineWidth', 1.5); grid on;

title('Butterworth LPF Phase Response');

xlabel('Frequency (Hz)'); ylabel('Phase (degrees)');

subplot(2,2,3);

plot(f, 20*log10(abs(H_hp)), 'b', 'LineWidth', 1.5); grid on;

title(['Butterworth HPF Magnitude (Order N = ', num2str(N_hp), ')']);

xlabel('Frequency (Hz)'); ylabel('Magnitude (dB)'); ylim([-80 5]);

subplot(2,2,4);

plot(f, angle(H_hp)*180/pi, 'r', 'LineWidth', 1.5); grid on;

title('Butterworth HPF Phase Response');

xlabel('Frequency (Hz)'); ylabel('Phase (degrees)');

OUTPUT:

=== EXPERIMENT 1: BUTTERWORTH IIR FILTERS ===

--- LOW-PASS FILTER ---
Calculated Filter Order (N_lp): 12
3-dB Cutoff Frequency (Wn_lp): 1087.8354 Hz
Numerator Coefficients (b_lp):
    0.0000    0.0000    0.0002    0.0006    0.0013    0.0021    0.0024    0.0021    0.0013    0.0006    0.0002    0.0000    0.0000

Denominator Coefficients (a_lp):
    1.0000   -5.4622   14.6771  -25.1616   30.3497  -26.9537   17.9862   -9.0534    3.4018   -0.9283    0.1743   -0.0202    0.0011

--- HIGH-PASS FILTER ---
Calculated Filter Order (N_hp): 10
3-dB Cutoff Frequency (Wn_hp): 2379.7556 Hz
Numerator Coefficients (b_hp):
    0.0006   -0.0055    0.0248   -0.0662    0.1159   -0.1390    0.1159   -0.0662    0.0248   -0.0055    0.0006

Denominator Coefficients (a_hp):
    1.0000    1.8915    2.8535    2.6014    1.8704    0.9534    0.3726    0.1030    0.0199    0.0023    0.0001