SIMULATION EXPERIMENT –10

Investigation of Coefficient and Product Quantization Noise and Limit-Cycle

Oscillations Using MATLAB

program

clc; clear; close all;

%% INPUTS

b = [0.0048 0.0193 0.0289 0.0193 0.0048];

a = [1 -2.3695 2.3140 -1.0547 0.1874];

Bc = 8; % Coefficient word length

Bp = 16; % Product word length

N = 200;

x = [1 zeros(1,N-1)]; % Unit impulse

%% 1. COEFFICIENT QUANTIZATION

S = 2^(Bc-1)-1;

bq = round(b*S)/S;

aq = round(a*S)/S;

CoeffError = [b-bq a-aq];

%% 2. IDEAL AND COEFFICIENT-QUANTIZED RESPONSE

y_ideal = filter(b,a,x);

y_coeff = filter(bq,aq,x);

e_coeff = y_ideal-y_coeff;

MaxCoeffError = max(abs(e_coeff));

SQNR_coeff = 10*log10(sum(y_ideal.^2)/sum(e_coeff.^2));

%% 3. PRODUCT QUANTIZATION

Sp = 2^(Bp-1)-1;

y_prod = zeros(1,N);

v = zeros(1,4);

for n = 1:N

p = bq*x(n);

fb = -aq(2:5).*v;

p = round([p fb]*Sp)/Sp; % Product quantization

y_prod(n) = sum(p);

v = [y_prod(n) v(1:3)];

end

e_prod = y_coeff-y_prod;

MaxProdError = max(abs(e_prod));

SQNR_prod = 10*log10(sum(y_coeff.^2)/sum(e_prod.^2));

%% 4. ZERO-INPUT LIMIT-CYCLE TEST

x0 = zeros(1,N);

state = [0.01 0.01 0.01 0.01];

y_zero = zeros(1,N);

for n = 1:N

acc = -aq(2:5)*state.&#39;;

y_zero(n) = round(acc*Sp)/Sp;

state = [y_zero(n) state(1:3)];

end

LimitAmplitude = max(abs(y_zero));

LimitSamples = find(abs(y_zero)&gt;1e-12);

%% 5. OVERFLOW / SATURATION TEST

xo = 1.2*ones(1,N);

y_over = filter(bq,aq,xo);

% Normalized saturation

y_sat = min(max(y_over,-1),1);

OverflowCount = sum(abs(y_over)&gt;1);

MaxOverflow = max(abs(y_over));

%% 6. OUTPUT TABLE

fprintf(&#39;\n--- QUANTIZATION RESULTS ---\n&#39;);

fprintf(&#39;Maximum coefficient/output error = %.6e\n&#39;,MaxCoeffError);

fprintf(&#39;Coefficient SQNR = %.2f dB\n&#39;,SQNR_coeff);

fprintf(&#39;Maximum product quantization error = %.6e\n&#39;,MaxProdError);

fprintf(&#39;Product SQNR = %.2f dB\n&#39;,SQNR_prod);

fprintf(&#39;Zero-input maximum amplitude = %.6e\n&#39;,LimitAmplitude);

fprintf(&#39;Overflow samples = %d\n&#39;,OverflowCount);

T = table( ...

[&quot;Coefficient Quantization&quot;;

&quot;Product Quantization&quot;;

&quot;Zero-Input Test&quot;;

&quot;Overflow Test&quot;], ...

[MaxCoeffError; MaxProdError; LimitAmplitude; MaxOverflow], ...

[SQNR_coeff; SQNR_prod; NaN; NaN], ...

&#39;VariableNames&#39;,{&#39;Test&#39;,&#39;Maximum_Error_or_Amplitude&#39;,&#39;SQNR_dB&#39;});

disp(T);

%% 7. QUANTIZED COEFFICIENTS

disp(&#39;Original b =&#39;); disp(b);

disp(&#39;Quantized b =&#39;); disp(bq);

disp(&#39;Original a =&#39;); disp(a);

disp(&#39;Quantized a =&#39;); disp(aq);

%% 8. PLOTS

figure;

stem(0:N-1,y_ideal,&#39;filled&#39;); hold on;

stem(0:N-1,y_coeff);

grid on; xlabel(&#39;n&#39;); ylabel(&#39;Amplitude&#39;);

title(&#39;Ideal vs Coefficient-Quantized Response&#39;);

legend(&#39;Ideal&#39;,&#39;Quantized&#39;);

figure;

plot(0:N-1,e_coeff,&#39;LineWidth&#39;,1);

grid on; xlabel(&#39;n&#39;); ylabel(&#39;Error&#39;);

title(&#39;Coefficient Quantization Noise&#39;);

figure;

stem(0:N-1,y_coeff-y_prod,&#39;filled&#39;);

grid on; xlabel(&#39;n&#39;); ylabel(&#39;Error&#39;);

title(&#39;Product Quantization Noise&#39;);

figure;

plot(0:N-1,y_zero,&#39;LineWidth&#39;,1);

grid on; xlabel(&#39;n&#39;); ylabel(&#39;Amplitude&#39;);

title(&#39;Zero-Input Limit-Cycle Test&#39;);

figure;

plot(0:N-1,y_over); hold on;

plot(0:N-1,y_sat,&#39;--&#39;);

grid on; xlabel(&#39;n&#39;); ylabel(&#39;Amplitude&#39;);

title(&#39;Overflow / Saturation Test&#39;);

legend(&#39;Without Saturation&#39;,&#39;With Saturation&#39;);

figure;

zplane(b,a); grid on;

title(&#39;Ideal Filter Pole-Zero Plot&#39;);

figure;

zplane(bq,aq); grid on;

title(&#39;Quantized Filter Pole-Zero Plot&#39;);