SIMULATION EXPERIMENT –7

Design and Analysis of FIR Filters Using Parks–McClellan and Frequency-Sampling

Techniques Using MATLAB

Program Code:

clear; clc; close all;

Fs = 10000;

fp = 1500;

fs = 2000;

N = 40; % Filter order

% 1. Parks-McClellan (Equiripple) Design

f_bands = [0, fp, fs, Fs/2] / (Fs/2); % Normalized frequency band edges

a_bands = [1, 1, 0, 0]; % Desired amplitudes

b_pm = firpm(N, f_bands, a_bands);

% 2. Frequency-Sampling Design

b_fs = fir2(N, f_bands, a_bands);

% ==========================================

% DISPLAY NUMERICAL OUTPUTS IN COMMAND WINDOW

% ==========================================

fprintf(&#39;=== EXPERIMENT 5: PARKS-MCCLELLAN vs FREQUENCY SAMPLING ===\n\n&#39;);

fprintf(&#39;Filter Order (N): %d\n&#39;, N);

fprintf(&#39;Passband Edge: %d Hz, Stopband Edge: %d Hz\n\n&#39;, fp, fs);

fprintf(&#39;--- PARKS-MCCLELLAN COEFFICIENTS (b_pm, First 10) ---\n&#39;);

disp(b_pm(1:10));

fprintf(&#39;--- FREQUENCY-SAMPLING COEFFICIENTS (b_fs, First 10) ---\n&#39;);

disp(b_fs(1:10));

% Frequency Responses

[H_pm, f] = freqz(b_pm, 1, 1024, Fs);

[H_fs, ~] = freqz(b_fs, 1, 1024, Fs);

% Plotting

figure(&#39;Name&#39;, &#39;Parks-McClellan vs Frequency-Sampling&#39;);

subplot(2,1,1);

plot(f, 20*log10(abs(H_pm)), &#39;b&#39;, &#39;LineWidth&#39;, 1.5); hold on;

plot(f, 20*log10(abs(H_fs)), &#39;r--&#39;, &#39;LineWidth&#39;, 1.5); grid on;

legend(&#39;Parks-McClellan (Equiripple)&#39;, &#39;Frequency-Sampling&#39;);

title([&#39;FIR Filter Magnitude Response Comparison (Order N = &#39;, num2str(N), &#39;)&#39;]);

xlabel(&#39;Frequency (Hz)&#39;); ylabel(&#39;Magnitude (dB)&#39;); ylim([-80 10]);

subplot(2,1,2);

plot(f, angle(H_pm)*180/pi, &#39;b&#39;, &#39;LineWidth&#39;, 1.5); hold on;

plot(f, angle(H_fs)*180/pi, &#39;r--&#39;, &#39;LineWidth&#39;, 1.5); grid on;

legend(&#39;Parks-McClellan&#39;, &#39;Frequency-Sampling&#39;);

title(&#39;Phase Response Comparison&#39;);

xlabel(&#39;Frequency (Hz)&#39;); ylabel(&#39;Phase (degrees)&#39;);