SIMULATION EXPERIMENT –8

Realization of FIR Filters Using Direct, Cascade and Linear-Phase Structures Using

MATLAB

program

clc; clear; close all;

%% FIR FILTER PARAMETERS

b = [0.0625 0.2500 0.3750 0.2500 0.0625];

Fs = 1000;

N = 128;

x = [1 zeros(1,N-1)];

n = 0:N-1;

%% 1. DIRECT REALIZATION

yd = filter(b,1,x);

%% 2. CASCADE REALIZATION

z = roots(b);

g = b(1);

yc = g*x;

for k = 1:length(z)

yc = filter([1 -z(k)],1,yc);

end

yc = real(yc);

%% 3. SYMMETRY AND LINEAR PHASE

symErr = max(abs(b-fliplr(b)));

D = (length(b)-1)/2;

if symErr &lt; 1e-12

symmetry = &#39;Symmetric&#39;;

else

symmetry = &#39;Not Symmetric&#39;;

end

%% 4. FREQUENCY, PHASE AND GROUP DELAY

[H,w] = freqz(b,1,1024);

phase = unwrap(angle(H));

gd = grpdelay(b,1,1024);

%% 5. NUMERICAL COMPARISON

err = max(abs(yd-yc));

dev = err/max(abs(yd))*100;

%% =========================================================

% DISPLAY RESULTS

% =========================================================

fprintf(&#39;\n============================================\n&#39;);

fprintf(&#39; FIR FILTER REALIZATION RESULTS\n&#39;);

fprintf(&#39;============================================\n&#39;);

fprintf(&#39;\nFIR Coefficients:\n&#39;);

disp(b);

fprintf(&#39;Sampling Frequency = %d Hz\n&#39;,Fs);

fprintf(&#39;Number of Samples = %d\n&#39;,N);

fprintf(&#39;Coefficient Symmetry = %s\n&#39;,symmetry);

fprintf(&#39;Symmetry Error = %.3e\n&#39;,symErr);

fprintf(&#39;Group Delay = %.2f samples\n&#39;,D);

fprintf(&#39;\nMaximum Error = %.3e\n&#39;,err);

fprintf(&#39;Percentage Deviation = %.3e %%\n&#39;,dev);

%% =========================================================

% IMPULSE RESPONSE VALUES

% =========================================================

fprintf(&#39;\n============================================\n&#39;);

fprintf(&#39;FIRST 10 IMPULSE RESPONSE VALUES\n&#39;);

fprintf(&#39;============================================\n&#39;);

fprintf(&#39; n Direct Cascade\n&#39;);

for k = 1:10

fprintf(&#39;%4d %10.6f %10.6f\n&#39;, ...

n(k),yd(k),yc(k));

end

%% =========================================================

% CASCADE ZEROS

% =========================================================

fprintf(&#39;\n============================================\n&#39;);

fprintf(&#39;CASCADE REALIZATION ZEROS\n&#39;);

fprintf(&#39;============================================\n&#39;);

disp(z);

%% =========================================================

% OUTPUT PARAMETER TABLE

% =========================================================

Parameter = {

&#39;Direct realization output&#39;

&#39;Cascade realization output&#39;

&#39;Impulse response&#39;

&#39;Phase response&#39;

&#39;Comparison of realizations&#39;

};

Observation = {

&#39;Direct FIR filtering&#39;

&#39;Cascade of first-order sections&#39;

&#39;Equal to FIR coefficient sequence&#39;

&#39;Linear phase&#39;

&#39;Equivalent transfer-function behavior&#39;

};

Value = {

sprintf(&#39;[%.4f %.4f %.4f %.4f %.4f]&#39;,yd(1:5))

sprintf(&#39;[%.4f %.4f %.4f %.4f %.4f]&#39;,yc(1:5))

sprintf(&#39;[%.4f %.4f %.4f %.4f %.4f]&#39;,yd(1:5))

sprintf(&#39;Group delay = %.2f samples&#39;,D)

sprintf(&#39;Max error = %.3e; Deviation = %.3e %%&#39;,err,dev)

};

Unit_Note = {

&#39;Amplitude&#39;

&#39;Amplitude&#39;

&#39;h[n]&#39;

&#39;Samples&#39;

&#39;Numerical precision&#39;

};

Output_Table = table(Parameter,Observation,Value,Unit_Note);

fprintf(&#39;\n============================================\n&#39;);

fprintf(&#39;OUTPUT PARAMETER TABLE\n&#39;);

fprintf(&#39;============================================\n&#39;);

disp(Output_Table);

%% =========================================================

% GRAPH 1: DIRECT VS CASCADE IMPULSE RESPONSE

% =========================================================

figure;

stem(n,yd,&#39;filled&#39;);

hold on;

stem(n,yc,&#39;--&#39;);

grid on;

xlabel(&#39;Sample n&#39;);

ylabel(&#39;Amplitude&#39;);

title(&#39;Direct and Cascade FIR Impulse Responses&#39;);

legend(&#39;Direct Realization&#39;,&#39;Cascade Realization&#39;);

%% =========================================================

% GRAPH 2: MAGNITUDE RESPONSE

% =========================================================

figure;

plot(w/pi,abs(H),&#39;LineWidth&#39;,1.5);

grid on;

xlabel(&#39;Normalized Frequency (\times\pi rad/sample)&#39;);

ylabel(&#39;|H(e^{j\omega})|&#39;);

title(&#39;FIR Magnitude Response&#39;);

%% =========================================================

% GRAPH 3: PHASE RESPONSE

% =========================================================

figure;

plot(w/pi,phase,&#39;LineWidth&#39;,1.5);

grid on;

xlabel(&#39;Normalized Frequency (\times\pi rad/sample)&#39;);

ylabel(&#39;Phase (radians)&#39;);

title(&#39;Unwrapped Phase Response&#39;);

%% =========================================================

% GRAPH 4: GROUP DELAY

% =========================================================

figure;

plot(w/pi,gd,&#39;LineWidth&#39;,1.5);

grid on;

xlabel(&#39;Normalized Frequency (\times\pi rad/sample)&#39;);

ylabel(&#39;Group Delay (samples)&#39;);

title(&#39;Group Delay&#39;);

%% =========================================================

% GRAPH 5: NUMERICAL ERROR

% =========================================================

figure;

plot(n,abs(yd-yc),&#39;LineWidth&#39;,1.5);

grid on;

xlabel(&#39;Sample n&#39;);

ylabel(&#39;Absolute Error&#39;);

title(&#39;Direct vs Cascade Numerical Error&#39;);

%% =========================================================

% FINAL RESULT

% =========================================================

fprintf(&#39;\n============================================\n&#39;);

fprintf(&#39;FINAL RESULT\n&#39;);

fprintf(&#39;============================================\n&#39;);

fprintf(&#39;1. FIR filter realized using Direct structure.\n&#39;);

fprintf(&#39;2. FIR filter realized using Cascade structure.\n&#39;);

fprintf(&#39;3. Coefficients are %s.\n&#39;,symmetry);

fprintf(&#39;4. Linear-phase group delay = %.2f samples.\n&#39;,D);

fprintf(&#39;5. Maximum realization error = %.3e.\n&#39;,err);

fprintf(&#39;6. Percentage deviation = %.3e %%.\n&#39;,dev);

fprintf(&#39;7. Direct and Cascade responses are equivalent.\n&#39;);