SIMULATION EXPERIMENT – 9

Uniform Quantization and Finite Word-Length Effects in Digital Signal Processing

Using MATLAB

Program

clc; clear; close all;

%% PARAMETERS

A=1; f=50; Fs=1000; N=1000;

n=0:N-1; x=A*sin(2*pi*f*n/Fs);

B=8; bits=[4 8 12 16];

%% 8-BIT QUANTIZATION

D=2*A/2^B;

xr=D*round(x/D); % Rounding

xt=D*fix(x/D); % Truncation

er=x-xr; % Rounding error

et=x-xt; % Truncation error

SQr=10*log10(mean(x.^2)/mean(er.^2));

SQt=10*log10(mean(x.^2)/mean(et.^2));

%% INPUT / OUTPUT QUANTIZATION

xq=xr;

y=x;

yq=D*round(y/D);

eo=y-yq;

SQout=10*log10(mean(y.^2)/mean(eo.^2));

%% WORD-LENGTH COMPARISON

for k=1:length(bits)

b=bits(k);

d=2*A/2^b;

qr=d*round(x/d);

qt=d*fix(x/d);

er1=x-qr; et1=x-qt;

step(k)=d;

sqnrR(k)=10*log10(mean(x.^2)/mean(er1.^2));

sqnrT(k)=10*log10(mean(x.^2)/mean(et1.^2));

maxER(k)=max(abs(er1));

maxET(k)=max(abs(et1));

end

%% RESULTS

fprintf(&#39;\n========== QUANTIZATION RESULTS ==========\n&#39;);

fprintf(&#39;Amplitude = %.1f | Frequency = %d Hz | Fs = %d Hz\n&#39;,A,f,Fs);

fprintf(&#39;Samples = %d | Main word length = %d bits\n&#39;,N,B);

fprintf(&#39;8-bit Step Size = %.7f\n&#39;,D);

fprintf(&#39;Rounding SQNR = %.4f dB\n&#39;,SQr);

fprintf(&#39;Truncation SQNR = %.4f dB\n&#39;,SQt);

fprintf(&#39;Output SQNR = %.4f dB\n&#39;,SQout);

fprintf(&#39;Max Rounding Error = %.6f\n&#39;,max(abs(er)));

fprintf(&#39;Max Truncation Error = %.6f\n&#39;,max(abs(et)));

%% FIRST 10 SAMPLES

T1=table(n(1:10)&#39;,x(1:10)&#39;,xr(1:10)&#39;,er(1:10)&#39;,xt(1:10)&#39;,et(1:10)&#39;,...

&#39;VariableNames&#39;,{&#39;n&#39;,&#39;Original&#39;,&#39;Rounded&#39;,&#39;RoundError&#39;,&#39;Truncated&#39;,&#39;TruncError&#39;});

disp(T1);

%% WORD-LENGTH TABLE

T2=table(bits&#39;,step&#39;,sqnrR&#39;,sqnrT&#39;,maxER&#39;,maxET&#39;,...

&#39;VariableNames&#39;,{&#39;Bits&#39;,&#39;StepSize&#39;,&#39;SQNR_Round_dB&#39;,...

&#39;SQNR_Trunc_dB&#39;,&#39;MaxError_Round&#39;,&#39;MaxError_Trunc&#39;});

disp(T2);

%% OUTPUT PARAMETER TABLE

Parameter={

&#39;Quantized signal&#39;

&#39;Quantization error&#39;

&#39;SQNR&#39;

&#39;Word-length comparison&#39;};

Observation={

sprintf(&#39;8-bit rounding, Delta = %.7f&#39;,D)

sprintf(&#39;Max rounding error = %.6f&#39;,max(abs(er)))

sprintf(&#39;Rounding = %.4f dB; Truncation = %.4f dB&#39;,SQr,SQt)

&#39;SQNR increases as word length increases&#39;};

Unit={

&#39;Amplitude&#39;

&#39;Amplitude&#39;

&#39;dB&#39;

&#39;Bits / dB&#39;};

disp(table(Parameter,Observation,Unit));

%% GRAPHS

figure;

plot(n(1:100),x(1:100)); hold on;

stairs(n(1:100),xr(1:100));

grid on; xlabel(&#39;Sample n&#39;); ylabel(&#39;Amplitude&#39;);

title(&#39;Original and 8-bit Quantized Signal&#39;);

legend(&#39;Original&#39;,&#39;Quantized&#39;);

figure;

plot(n,er); hold on; plot(n,et);

grid on; xlabel(&#39;Sample n&#39;); ylabel(&#39;Error&#39;);

title(&#39;Rounding and Truncation Quantization Error&#39;);

legend(&#39;Rounding&#39;,&#39;Truncation&#39;);

figure;

plot(bits,sqnrR,&#39;o-&#39;); hold on;

plot(bits,sqnrT,&#39;s--&#39;);

grid on; xlabel(&#39;Word Length (bits)&#39;); ylabel(&#39;SQNR (dB)&#39;);

title(&#39;SQNR vs Word Length&#39;);

legend(&#39;Rounding&#39;,&#39;Truncation&#39;);

figure;

semilogy(bits,maxER,&#39;o-&#39;); hold on;

semilogy(bits,maxET,&#39;s--&#39;);

grid on; xlabel(&#39;Word Length (bits)&#39;);

ylabel(&#39;Maximum Quantization Error&#39;);

title(&#39;Quantization Error vs Word Length&#39;);

legend(&#39;Rounding&#39;,&#39;Truncation&#39;);

%% FINAL RESULT

fprintf(&#39;\n========== FINAL RESULT ==========\n&#39;);

fprintf(&#39;Uniform quantization using rounding and truncation was performed.\n&#39;);

fprintf(&#39;Quantization error and SQNR were calculated.\n&#39;);

fprintf(&#39;Increasing word length reduces quantization error and im

proves SQNR.\n&#39;);

fprintf(&#39;Finite word-length effects are therefore verified.\n&#39;);