SIMULATION EXPERIMENT –6

Design and Analysis of FIR Digital Filters Using Fourier Series and Windowing

Techniques Using MATLAB

Program Code:

clear; clc; close all;

Fs = 8000;

fc = 1500;

wc = fc / (Fs/2); % Normalized cutoff frequency

N = 50; % Filter order

% Window Functions

w_rect = rectwin(N+1);

w_hamm = hamming(N+1);

w_hann = hann(N+1);

w_black = blackman(N+1);

% FIR Filter Design using fir1

b_rect = fir1(N, wc, w_rect);

b_hamm = fir1(N, wc, w_hamm);

b_hann = fir1(N, wc, w_hann);

b_black = fir1(N, wc, w_black);

% Frequency Response Calculation

[H_rect, f] = freqz(b_rect, 1, 1024, Fs);

[H_hamm, ~] = freqz(b_hamm, 1, 1024, Fs);

[H_hann, ~] = freqz(b_hann, 1, 1024, Fs);

[H_black, ~] = freqz(b_black, 1, 1024, Fs);

% ==========================================

% DISPLAY NUMERICAL OUTPUTS IN COMMAND WINDOW

% ==========================================

fprintf(&#39;=== EXPERIMENT 4: FIR WINDOWING TECHNIQUES ===\n\n&#39;);

fprintf(&#39;Filter Order (N): %d\n&#39;, N);

fprintf(&#39;Cutoff Frequency: %d Hz\n\n&#39;, fc);

fprintf(&#39;--- RECTANGULAR WINDOW COEFFICIENTS (First 5) ---\n&#39;);

disp(b_rect(1:5));

fprintf(&#39;--- HAMMING WINDOW COEFFICIENTS (First 5) ---\n&#39;);

disp(b_hamm(1:5));

fprintf(&#39;--- HANNING WINDOW COEFFICIENTS (First 5) ---\n&#39;);

disp(b_hann(1:5));

fprintf(&#39;--- BLACKMAN WINDOW COEFFICIENTS (First 5) ---\n&#39;);

disp(b_black(1:5));

% Plotting

figure(&#39;Name&#39;, &#39;FIR Windowing Techniques Comparison&#39;);

subplot(2,1,1);

plot(f, 20*log10(abs(H_rect)), &#39;k&#39;, &#39;LineWidth&#39;, 1); hold on;

plot(f, 20*log10(abs(H_hamm)), &#39;b&#39;, &#39;LineWidth&#39;, 1.5);

plot(f, 20*log10(abs(H_hann)), &#39;g&#39;, &#39;LineWidth&#39;, 1.5);

plot(f, 20*log10(abs(H_black)), &#39;r&#39;, &#39;LineWidth&#39;, 1.5); grid on;

legend(&#39;Rectangular&#39;, &#39;Hamming&#39;, &#39;Hanning&#39;, &#39;Blackman&#39;);

title(&#39;FIR Low-Pass Filter Magnitude Response (N = 50)&#39;);

xlabel(&#39;Frequency (Hz)&#39;); ylabel(&#39;Magnitude (dB)&#39;); ylim([-100 10]);

subplot(2,1,2);

plot(f, angle(H_hamm)*180/pi, &#39;b&#39;, &#39;LineWidth&#39;, 1.5); grid on;

title(&#39;Linear Phase Response (Hamming Window Example)&#39;);

xlabel(&#39;Frequency (Hz)&#39;); ylabel(&#39;Phase (degrees)&#39;);